Networking   5x

Commands;

mrload

setAdoption

Start up system

Type b enter

Chose the site you are working at

Type n to stop genesis auto boot

Or if system is already booted up then hit L1 and B

Type: login

Genesis

4$apps

At prompt type: sd   -i

You now should be at a promt with applications shut down

Login as root

Password #bigguy

Type mrload

Be really careful with what you answer to the questions because you can mess up software. Accept the defaults for most answers. You should be able to find the MR IP address and AE title in the questions.

After you are done it should reset

It will then ask you to login to the site twice

Then when it gets to the prompt

login as genesis

Password 4$apps

Then type st

to start apps

setAdoption

use c-shell
su for root, #bigguy for password
setAdoption
select 2 for idnet.
unadopt suite, or enter suiteID, then hostID (these are only seen by 5x).
enter AE title (arcnema)
enter IP address
enter port number
then enter, enter, enter.
then exit, exit.
host is now listed under. network, manual transmit.